APPENDIX 1.  Code to drive various graphics devices.

Some of this code was supplied by David L. Swofford and

Julian M. Humphries.  Besides adding the code below, you will have

to add the plotter name to the list of names declared for the type

"plottertype" at the beginning of the program.  Many of these

plotters are already supported by the programs as I distribute

them.  If you are writting code for another graphics device, you

can copy these examples extensively.

Hewlett Packard plotter:

This code will drive a large number of plotters including the

IBM pen plotters.  This code is included in the programs as I

distribute them.

Code to insert in procedure plotrparms:

'H': BEGIN

plotter := hp7470;

penchange := yes;

xunitspercm := 400.0;

yunitspercm := 400.0;

xsize := 24.0;

ysize := 18.0;

xposition := 12.89;

yposition := 9.905;

xoption := center;

yoption := center;

rotation := 0.0;

END;

Code to insert in procedure plot:

hp7470: BEGIN

IF penstatus=pendown

THEN WRITE(plotfile, 'PD')

ELSE WRITE(plotfile, 'PU');

pout(ROUND(xabs));

WRITE(plotfile, ',');

pout(ROUND(yabs));

WRITELN(plotfile, ';');

END;

Code to insert in procedure initplotter:

hp7470: WRITELN(plotfile, 'IN;SP1;VS10.0;');

Code to insert in procedure finishplotter:

hp7470: BEGIN

plot(penup,0.0,0.0);

WRITELN(plotfile,'SP;');

END;

Calcomp plotters:

Code to insert in procedure plotrparms:

'C': BEGIN

plotter := calcomp;

xunitspercm := 39.37;

yunitspercm := 39.37;

xsize := 25.0;

ysize := 25.0;

xposition := 12.5;

yposition := 0.0;

xoption := center;

yoption := above;

rotation := 0.0;

END;

Code to insert in procedure plot:

calcomp: BEGIN

IF penstatus = pendown

THEN WRITE(plotfile,'H')

ELSE WRITE(plotfile,'D');

xrel := ROUND(xabs) - xnow;

yrel := ROUND(yabs) - ynow;

xnow := ROUND(xabs);

ynow := ROUND(yabs);

IF xrel > 0

THEN IF yrel > 0

THEN quadrant := 'P'

ELSE quadrant := 'T'

ELSE IF yrel > 0

THEN quadrant := 'X'

ELSE quadrant := '1';

xrel := ABS(xrel);

yrel := ABS(yrel);

IF xrel > yrel

THEN n := (xrel DIV 255) + 1

ELSE n := (yrel DIV 255) + 1;

xinc := xrel DIV n;

yinc := yrel DIV n;

xlast := xrel MOD n;

ylast := yrel MOD n;

xhigh := xinc DIV 16;

yhigh := yinc DIV 16;

xlow := xinc MOD 16;

ylow := yinc MOD 16;

FOR i := 1 TO n DO

WRITE(plotfile, quadrant, cchex[xhigh], cchex[xlow],

cchex[yhigh], cchex[ylow]);

IF (xlast <> 0) OR (ylast <> 0)

THEN WRITE(plotfile, quadrant, cchex[0], cchex[xlast],

cchex[0], cchex[ylast]);

END;

Code to insert in procedure initplotter:

calcomp: BEGIN

cchex[0] := 'C'; cchex[1] := 'D'; cchex[2] := 'H';

cchex[3] := 'L'; cchex[4] := 'P'; cchex[5] := 'T';

cchex[6] := 'X'; cchex[7] := '1'; cchex[8] := '5';

cchex[9] := '9'; cchex[10] := '/'; cchex[11] := '=';

cchex[12] := '#'; cchex[13] := '"'; cchex[14] := '''';

cchex[15] := '^';

xnow := 0;

ynow := 0;

WRITE(plotfile, 'CCCCCCCCCC');

END;

Code to insert in procedure finishplotter:

calcomp: plot(penup, 0.0, yrange+50.0);

Tektronix plotters:

This code was heavily modified by David L. Swofford and

Julian M. Humphries.

Place this declaration at the beginning of the program if you are

using EBCDIC:

eb : ARRAY[0..127] OF INTEGER;

Place this code at the beginning of the main program if you are

using EBCDIC:

escape := CHR(39);      {EBCDIC}    {DLS/JMH}

{ DLS/JMH -- ASCII to EBCDIC translation table.  Allows use of the

Tektronix driver on IBM and other machines that use EBCDIC.  }

eb[  0]:=  0; eb[  1]:=  1; eb[  2]:=  2; eb[  3]:=  3; eb[  4]:= 55;

eb[  5]:= 45; eb[  6]:= 46; eb[  7]:= 47; eb[  8]:= 22; eb[  9]:=  5;

eb[ 10]:= 37; eb[ 11]:= 11; eb[ 12]:= 12; eb[ 13]:= 13; eb[ 14]:= 14;

eb[ 15]:= 15; eb[ 16]:= 16; eb[ 17]:= 17; eb[ 18]:= 18; eb[ 19]:= 19;

eb[ 20]:= 60; eb[ 21]:= 61; eb[ 22]:= 50; eb[ 23]:= 38; eb[ 24]:= 24;

eb[ 25]:= 25; eb[ 26]:= 63; eb[ 27]:= 39; eb[ 28]:= 28; eb[ 29]:= 29;

eb[ 30]:= 30; eb[ 31]:= 31; eb[ 32]:= 64; eb[ 33]:= 90; eb[ 34]:=127;

eb[ 35]:=123; eb[ 36]:= 91; eb[ 37]:=108; eb[ 38]:= 80; eb[ 39]:=125;

eb[ 40]:= 77; eb[ 41]:= 93; eb[ 42]:= 92; eb[ 43]:= 78; eb[ 44]:=107;

eb[ 45]:= 96; eb[ 46]:= 75; eb[ 47]:= 97; eb[ 48]:=240; eb[ 49]:=241;

eb[ 50]:=242; eb[ 51]:=243; eb[ 52]:=244; eb[ 53]:=245; eb[ 54]:=246;

eb[ 55]:=247; eb[ 56]:=248; eb[ 57]:=249; eb[ 58]:=122; eb[ 59]:= 94;

eb[ 60]:= 76; eb[ 61]:=126; eb[ 62]:=110; eb[ 63]:=111; eb[ 64]:=124;

eb[ 65]:=193; eb[ 66]:=194; eb[ 67]:=195; eb[ 68]:=196; eb[ 69]:=197;

eb[ 70]:=198; eb[ 71]:=199; eb[ 72]:=200; eb[ 73]:=201; eb[ 74]:=209;

eb[ 75]:=210; eb[ 76]:=211; eb[ 77]:=212; eb[ 78]:=213; eb[ 79]:=214;

eb[ 80]:=215; eb[ 81]:=216; eb[ 82]:=217; eb[ 83]:=226; eb[ 84]:=227;

eb[ 85]:=228; eb[ 86]:=229; eb[ 87]:=230; eb[ 88]:=231; eb[ 89]:=232;

eb[ 90]:=233; eb[ 91]:=173; eb[ 92]:=224; eb[ 93]:=189; eb[ 94]:= 95;

eb[ 95]:=109; eb[ 96]:=121; eb[ 97]:=129; eb[ 98]:=130; eb[ 99]:=131;

eb[100]:=132; eb[101]:=133; eb[102]:=134; eb[103]:=135; eb[104]:=136;

eb[105]:=137; eb[106]:=145; eb[107]:=146; eb[108]:=147; eb[109]:=148;

eb[110]:=149; eb[111]:=150; eb[112]:=151; eb[113]:=152; eb[114]:=153;

eb[115]:=162; eb[116]:=163; eb[117]:=164; eb[118]:=165; eb[119]:=166;

eb[120]:=167; eb[121]:=168; eb[122]:=169; eb[123]:=192; eb[124]:= 79;

eb[125]:=208; eb[126]:=161; eb[127]:=  7;

Place this at the beginning of the main program if you are using

ASCII:

escape := CHR(27);        {ASCII }    {DLS/JMH}

Code to insert in procedure plotrparms:

'T': BEGIN

plotter := tektronix;

xunitspercm := 50.0;

yunitspercm := 50.0;

xsize := 20.46;

ysize := 15.6;

xposition := 10.23;

yposition := 7.8;

xoption := center;

yoption := center;

rotation := 0.0;

END;

Code to insert in procedure plot:

tektronix: BEGIN

IF penstatus = penup

THEN WRITE(plotfile, CHR(29));

xnow := ROUND(xabs);

ynow := ROUND(yabs);

xhigh := xnow DIV 32;

yhigh := ynow DIV 32;

xlow := xnow MOD 32;

ylow := ynow MOD 32;

{DLS/JMH -- begin code for systems that use ASCII coding}

IF yhigh <> oldyhigh THEN WRITE(plotfile,

CHR(yhigh+32));

IF (ylow <> oldylow) OR (xhigh <> oldxhigh)

THEN WRITE(plotfile, CHR(ylow+96));

IF xhigh <> oldxhigh THEN WRITE(plotfile,

CHR(xhigh+32));

WRITE(plotfile, CHR(xlow+64));

{DLS/JMH -- end code for systems that use ASCII coding}

{DLS/JMH -- begin code for systems that use EBCDIC coding}

{

IF yhigh <> oldyhigh THEN WRITE(plotfile,

CHR(eb[yhigh+32]));

IF (ylow <> oldylow) OR (xhigh <> oldxhigh)

THEN WRITE(plotfile, CHR(eb[ylow+96]));

IF xhigh <> oldxhigh THEN WRITE(plotfile,

CHR(eb[xhigh+32]));

WRITE(plotfile, CHR(eb[xlow+64]));

}

{DLS/JMH -- end code for systems that use EBCDIC coding}

oldxhigh := xhigh;

oldxlow := xlow;

oldyhigh := yhigh;

oldylow := ylow;

{ DLS/JMH -- Put CR-LFs in appropriate places.  This is }

{            needed if output is going to a file or if  }

{            the terminal is treated as a file with a   }

{            maximum record length.                     }

nmoves := nmoves + 1;

IF nmoves = 12 THEN BEGIN

nmoves := 0;

WRITELN(plotfile)

END

{ DLS/JMH -- End code for CR-LFs }

END;

Code to insert in procedure initplotter:

tektronix: BEGIN

oldxhigh := -1;

oldxlow := -1;

oldyhigh := -1;

oldylow := -1;

nmoves := 0;  {DLS/JMH -- See Procedure PLOT}

{ DLS/JMH -- "escape" in next statement was originally      }

{            CHR(27), but we need to handle EBCDIC as       }

{            well as ASCII.  CHR(12) is same in both codes. }

WRITE(plotfile, escape,  CHR(12));   {DLS/JMH}

END;

Code to insert in procedure finishplotter:

tektronix: WRITELN(plotfile);

Bausch and Lomb plotters:

Code to insert in procedure plotrparms:

'B': BEGIN

plotter := bauschlomb;

penchange := yes;

xunitspercm := 100.0;

yunitspercm := 100.0;

xsize := 24.5;

ysize := 17.5;

xposition := 12.5;

yposition := 9.0;

xoption := center;

yoption := center;

rotation := 0.0;

END;

Code to insert in procedure plot:

bauschlomb: BEGIN

IF penstatus = pendown

THEN WRITE(plotfile, 'D ')

ELSE WRITE(plotfile, 'U ');

pout(ROUND(xabs));

WRITE(plotfile, ',');

pout(ROUND(yabs));

WRITELN(plotfile);

END;

Code to insert in procedure initplotter:

Nothing is necessary.

Code to insert in procedure finishplotter:

Nothing is necessary.

IBM Color Graphics Adapter:

This code was supplied by David L. Swofford and Julian M.

Humphries.  It requires the Turbo Pascal compiler.

Code to insert in procedure plotrparms:

'S': BEGIN { Code for direct output to PC screen }

plotter := screen;

penchange := no;

xunitspercm := 25.6;   { Based on 640 x 200 resolution  }

yunitspercm := 11.42;  { and a 25 cm x 17.5 cm physical }

xsize := 24.0;         { screen size.                   }

ysize := 16.0;

xposition := 12.5;

yposition := 8.75;

xoption := center;

yoption := center;

rotation := 0.0;

END;

Code to insert in procedure plot:

screen : BEGIN

newx := ROUND(xabs);

newy := ABS(ROUND(yabs)-199);

IF penstatus = pendown THEN

Draw(oldx,oldy,newx,newy,1);

oldx := newx;

oldy := newy

END;

Code to insert in procedure initplotter:

screen : BEGIN

HiRes;

oldx := 300;

oldy := 1;

END;

Code to insert in procedure finishplotter:

screen : BEGIN

READ(KBD, trash);

TextMode;

END;

IBM Enhanced Graphics Adapter:

This code was supplied by David L. Swofford and Julian M.

Humphries.  It requires the Turbo Pascal compiler.

Put this code before the statement 'PROCEDURE plot':

{this program uses the high resolution mode, 640X350 by 16 color graphics

that are available with the enhanced graphics adapter and enhanced color

monitor.  This was written by Alan Curtis, 1985.                        }

procedure super_hires;

{set resolution to 640X350 by 16 color mode}

type

result = record

al,ah,bl,bh : byte;

cx,dx,bp,si,di,ds,es,flags : integer;

end;

var

int_result : result;

col : byte;

begin

with int_result do begin

al := $10;  {graphics mode}

ah := $0;

end;

intr($10,int_result);  {high res mode}

end;

{////////////////////////////}

procedure set_page(page : integer);

{sets active display page - must have 128K of graphics memory}

type

result = record

al,ah,bl,bh : byte;

cx,dx,bp,si,di,ds,es,flags : integer;

end;

var

int_result : result;

b_page : byte;

begin

b_page := page;

int_result.ah := $5;

int_result.al := b_page;  {active page}

intr($10,int_result);

end;

procedure plot_line(x1,y1,x2,y2 : real; color,page : integer);

{draws a line from x1,y1 to x2,y2 in the specified color}

type

result = record

al,ah,bl,bh : byte;

cx,dx,bp,si,di,ds,es,flags : integer;

end;

var

counter : real;

int_result : result;

dx,dy,ddx,ddy,newx,newy : real;

b_page,b_color : byte;

xmult,ymult : integer;

begin

b_page := page;

b_color := color;

int_result.ah := $C;

int_result.bh := b_page;  {active page}

int_result.al := b_color;

int_result.dx := round(y1);

int_result.cx := round(x1);

intr($10,int_result);

dx := x2 - x1;

if dx < 0.0 then xmult := -1 else xmult := 1;  {increment can be neg.}

dy := y2 - y1;

if dy < 0.0 then ymult := -1 else ymult := 1;

{before getting ratios, check for zeros in dx and dy}

if dy = 0.0 then begin

{x will be incremented by 1 through every loop, y by 0}

ddx := 1.0 * xmult;

ymult := 0;

dy := abs(dx);    {fool loop counter below}

end

else if dx = 0.0 then begin

ddx := 2;

ddy := 1 * ymult;

xmult := 0;

dx := abs(dy);    {fool loop counter}

end

else begin

ddx := abs(dx/dy);  {get ratio}

ddy := abs(dy/dx);

ddx := ddx*xmult;

ddy := ddy*ymult;

end;

{the following initializations are used in the proceeding loops}

newy := y1;

newx := x1;

counter := 0.0;

dy := abs(dy);

dx := abs(dx);

if abs(ddx) <= 1.0 then

while counter < dy do begin

counter := counter + 1.0;

newx := newx + ddx;

newy := newy + ymult;

int_result.ah := $C;

int_result.bh := b_page;  {active page}

int_result.al := b_color;

int_result.dx := round(newy);

int_result.cx := round(newx);

intr($10,int_result);

end

else while counter < dx do begin  {x will be incremented by 1}

counter := counter + 1.0;

newx := newx + xmult;

newy := newy + ddy;

int_result.ah := $C;

int_result.bh := b_page;  {active page}

int_result.al := b_color;

int_result.dx := round(newy);

int_result.cx := round(newx);

intr($10,int_result);

end;

end;

Code to insert in procedure plotrparms:

'E': BEGIN { Code for IBM PC Enhanced Graphics Adaptor }

plotter := ega;

penchange := no;

xunitspercm := 25.6;   { Based on 640 x 350 resolution  }

yunitspercm := 20.0;   { and a 25 cm x 17.5 cm physical }

xsize := 24.0;         { screen size.                   }

ysize := 16.0;

xposition := 12.5;

yposition := 8.75;

xoption := center;

yoption := center;

rotation := 0.0;

END;

Code to insert in procedure plot:

ega : BEGIN

newx := ROUND(xabs);

newy := ABS(ROUND(yabs)-349);

IF penstatus = pendown THEN

plot_line(oldx,oldy,newx,newy,14,page);

oldx := newx;

oldy := newy

END;

Code to insert in procedure initplotter:

ega : BEGIN

Super_HiRes;

page := 0;

set_page(page);

GraphBackground(1);

oldx := 300;

oldy := 1;

END;

Code to insert in procedure finishplotter:

ega : BEGIN

READ(KBD, trash);

TextMode;

END;

IBM Professional Graphics Adapter:

This code requires the Turbo Pascal compiler.

This code must be placed before the statement 'PROCEDURE plot':

PROCEDURE wg(s : stype);

TYPE buffer = ARRAY [0..256] OF CHAR;

VAR i : INTEGER;

outbuff : buffer ABSOLUTE $C000:$6000;

wplace : CHAR ABSOLUTE $C000:$6300;

rplace : CHAR ABSOLUTE $C000:$6301;

BEGIN (* wg *)

s := s+' ';

FOR i := 1 TO LENGTH(s) DO BEGIN

WHILE (ORD(wplace)+1) MOD 256=ORD(rplace) DO BEGIN END;

outbuff[ORD(wplace)] := s[i];

wplace := CHR((ORD(wplace)+1) MOD 256);

END;

END; (* wg *)

PROCEDURE wg2(s: stype; n1, n2 : INTEGER);

VAR ns1, ns2 : STRING[10];

BEGIN (* wg2 *)

STR(n1, ns1);

STR(n2, ns2);

wg(s+' '+ns1+' '+ns2);

END; (* wg2 *)

PROCEDURE wg1(s: stype; n : INTEGER);

VAR ns : STRING[10];

BEGIN (* wg1 *)

STR(n, ns);

wg(s+' '+ns);

END; (* wg1 *)

Code to insert in procedure plotrparms:

'P': BEGIN { Code for IBM PC Professional Graphics }

plotter := prof;

penchange := yes;

xunitspercm := 26.6;   { Based on 640 x 480 resolution  }

yunitspercm := 26.6;   { and a 24 cm x 18 cm physical   }

xsize := 23.5;         { screen size.                   }

ysize := 17.5;

xposition := 0.0;

yposition := 0.0;

xoption := center;

yoption := center;

rotation := 0.0;

END;

Code to insert in procedure plot:

prof : BEGIN

IF penstatus = pendown

THEN wg2('D', ROUND(xabs), ROUND(yabs))

ELSE wg2('M', ROUND(xabs), ROUND(yabs));

END;

Code to insert in procedure initplotter:

prof : BEGIN

wg('CA');

wg('DI 0');

wg('LI 0');

wg('RF');

wg('L 0 0 0 10');

wg('L 1 15 15 15');

wg('L 2 15 15 15');

wg('CLS 0');

wg('C 1');

END;

Code to insert in procedure finishplotter:

Nothing is necessary.

ReGIS Graphics Protocol:

This code was supplied by David L. Swofford and Julian M.

Humphries.

Put this statement at the beginning of the main program:

escape := CHR(27);        {ASCII }    {DLS/JMH}

Code to insert in procedure plotrparms:

'R': BEGIN { Code for DEC's ReGIS protocol }

plotter := regis;

penchange := no;

xunitspercm := 1.0;    { Rather than attempt to use   }

yunitspercm := 1.0;    { physical device units, which }

xsize := 750.0;        { depend on the monitor being  }

ysize := 450.0;        { used, these units are in the }

xposition := 399.0;    { 800 x 480 coordinate system. }

yposition := 239.0;    { This works fine since the    }

xoption := center;     { DEC ReGIS protocol uses a    }

yoption := center;     { one-to-one aspect ratio.     }

rotation := 0.0;

END;

Code to insert in procedure plot:

regis : BEGIN { DEC's ReGIS graphics (VT125, VT240, VT241) }

newx := ROUND(xabs);

newy := ABS(ROUND(yabs)-479);

IF penstatus = pendown THEN BEGIN

WRITE(plotfile, 'P['); pout(oldx);

WRITE(plotfile, ','); pout(oldy);

WRITE(plotfile, ']V['); pout(newx);

WRITE(plotfile, ','); pout(newy);

WRITE(plotfile,']');

nmoves := nmoves + 1;

IF nmoves = 3 THEN BEGIN

nmoves := 0;

WRITELN(plotfile)

END

END;

oldx := newx;

oldy := newy

END;

Code to insert in procedure initplotter:

regis : BEGIN

oldx := 300;

oldy := 1;

nmoves := 0;

WRITE(plotfile,escape,'[2J');          {Clear screen}

WRITE(plotfile,escape,'Pp');           {Enter ReGIS }

WRITE(plotfile,'W(I3)');               {Set tree color}

WRITELN(plotfile,'S(A[0,0][799,479])') {Define addressing}

END;

Code to insert in procedure finishplotter:

regis : BEGIN

WRITELN(plotfile,escape,'')

END;

=========================================================================

=========================================================================

ADDENDUM by Joe Felsenstein

(c) Copyright 1986 by the University of Washington.  Permission is granted

to copy this document provided that no fee is charged for it and that this

copyright notice is not removed.

In the PHYLIP distribution version of these programs, I

have disabled the FONT command, as it is hard to implement it in a

general way.  I have left in all the code for FONT but it never

gets called from the main program.  Users wishing to restore FONT

can take the relevant code from the Turbo Pascal main program

supplied at the end of the document for each program.

Everyone else can use different fonts in a straightforward

way without specifying them in the FONT command.  Simply keep the

different font files, and before you run the program, simply make

sure that the font file you want to use is copied into the one the

program reads.  Thus if you have files called (say) FONT1, FONT2,

..., FONT5, have the program always read its font from a file

called FONTFILE.  Before the run simply copy the font you want

(say FONT3) into FONTFILE, so that the program, when it reads its

font from FONTFILE, reads the one you want.  The only changes you

will have to make in the distribution version of the program are

to modify the RESET(fontfile) statement or put an ASSIGN statement

before it, exactly as you have to do for the other RESET and

REWRITE statements.

GENERAL COMMENTS ON GETTING STARTED

It's a lot easier than Chris makes it sound in the

documentation above.  First you get the programs compiled,

which is mostly the same as getting anything else in the

package compiled and running, keeping in mind mostly that these

programs have three input files (parmfile, treefile, and

fontfile) and one output file (plotfile) which will probably

need a CLOSE statement so it doesn't get lost.

OK, say that's done.  How do you give all the commands to

run the programs.  Gosh, it sounds awfully complicated, doesn't

it?  It isn't!  Just make up the treefile containing some tree,

say:

((A,B),(C,D));

and a parmfile that contains just one line (trust the defaults for

now)

PLOTTER H

(or whatever of the plotters you have: T for Tektronix, B for

Bausch and Lomb, or C for Calcomp (we still don't know if that

one works)).

Now for the font file.  Try copying FONT2 (say) into your

fontfile.

Now run it!  If it seems to have worked, try sending the

contents of plotfile to your plotter.

If it has trouble reading the treefile or parmfile and gets

upset and complains about end-of-lines or end-of-files, you

probably have one line too many in that file.  These programs are

rather sensitive to that.  Try deleting the blank line after the

last substantive line in these files. (Or adding one if it wants

it that way).

If you have got out one of these (beautiful) plots, now you

are ready to read everything more carefully and start playing

around with the parmfile and making things more complicated.  Have

a look at LABELSIZE, XSIZE, YSIZE, XPOSITION, YPOSITION, ROTATION,

etc.

I must simply add that PLOTREE and PLOTGRAM are absolutely

beautiful programs that produce marvellous results, absolutely

publishable.  Try them -- you'll like them.

==================================================================

==================================================================

DOTREE and DOTGRAM   --- all you need to know.

by Joe Felsenstein

(note:  DOTREE pronounced DOT-TREE!)

These are versions of PLOTREE and PLOTGRAM that I have prepared by

modifying PLOTREE and PLOTGRAM.  They work on dot matrix printers.

The printers supported at the moment are:

Epson printers with dot graphics, including all sorts of

printers that are Epson-compatible in their graphics,

such as Star Gemini family, Panasonic KP1091 etc.

C. Itoh Prowriter and family -- including its sibling the

NEC 8023 (and 8025), and the printers by TEC (Tokyo

Electric) who actually are the ones that produce these

printers.

Toshiba P1350, P1351, P351, P1340, P341, etc.

Okidata Microline ML family.

The programs take a few minutes to produce the graphics information

in a file, which you then copy to your printer.  I have to say that

the printing speed of the Star Gemini and C. Itoh printers I tried

was acceptable, but that the Toshiba printers, which are very fancy

24-dot-per-character printers, simply had so many little dots to

dump to the printer that at 1200 baud it took about 20 minutes for

one plot.

One caution -- some operating systems change bytes as they go out

to the printer.   You have to stop them from doing that.  Unix may

give you trouble with this.  Ask a "guru".   For MSDOS (IBM PCDOS)

you will want to COPY the file to your printer device (whatever that

is) but with a /B after the COPY so that the operating system is

properly intimidated and does not mess with your file.  On CP/M-80

you can use (if your printer is hooked to the LST: device) and the

file with all the little dots in it is called PLOTFILE :

PIP LST:=PLOTFILE[O]

the [O] being the trick that intimidates the operating system.

FEATURES

Almost identical to PLOTREE and PLOTGRAM!  With the following

exceptions:

(1)  No CHANGEPEN command -- because there is no pen to change!

(2)  You say PRINTER instead of PLOTTER.  This I did so you can't

mistakenly call PLOTGRAM when you mean to call DOTGRAM and not be

stopped by an error message.  The printer symbols are:

Epson compatible     E

C. Itoh family       C

Toshibas             T

Okidata              K

(3)  The parameter defaults are somewhat different.  See

the detailed code of the PROCEDUREs initparms and printrparms

to get the details, if needed.

USING THE PROGRAMS

Same as PLOTREE and PLOTGRAM, with the exceptions noted.

HOW THEY WORK

Same as PLOTREE and PLOTGRAM for working out tree

coordinates.  When it comes the actual printing, the program works

down your page one strip one character deep at a time.  For each

strip the program goes through the tree and figures out if any

dots in that strip need to be turned on.  When it finishes drawing

in the bunch of bytes that represent all the dots in that strip,

the routine striprint prints the strip out, and goes on to the next

strip.  Every five strips a number (5, 10, 15, ...) is printed out

just to reassure you.  When all is finished, the plot file needs

to be dumped to your printer.

HELP!

You can help me by letting me know graphics details of any

other printer so we can include the code or append it to this

documentation.

SPECIFIC INSTRUCTIONS FOR EACH PROGRAM

There aren't any.   Read the instructions for PLOTREE and/or

PLOTGRAM, then change PLOTTER to PRINTER, provide the proper

printer code (E, C, T, or K) and go to it.